HUNTER Freeze-Clik

Freeze Sensor

The freeze sensor shall be capable of interrupting the power from the irrigation controller to the valves when ambient air temperature falls below 37 degrees Fahrenheit (3 degrees Centigrade).

The freeze sensor circuitry shall be housed in a UV and corrosion resistant plastic casing and the sensing element shall be encased in epoxy. The switch shall be rated at 24 VAC, 6 amps.

When specified as reverse switching, the unit will open below 37 degrees Fahrenheit (3 degrees Centigrade).

The sensor shall be installed in accordance with the manufacturer’s published instructions. The sensor shall carry a conditional five-year exchange warranty. The freeze sensor shall be the Freeze-Clik series as manufactured by Hunter Industries Incorporated, San Marcos, California.